# BTEC Applied Science Unit 5 Summer Transition Booklet

# Physics Section

## From OCR GCSE Combined Science to Level 3 Applied Science

This booklet develops GCSE Physics knowledge into the greater depth required for Unit 5.

You will study:

- Thermal Physics

- Specific Heat Capacity

- Work Done

- Efficiency

- Pressure and Volume

- Stress-Strain Curves

- Material Properties

- Elasticity and Plasticity

- Creep and Fatigue

Each topic includes:

- GCSE Retrieval Tasks

- Revision Notes

- Knowledge Checks

- Application Questions

- Exam-Style Questions

- Worked Examples

- Extended Response Questions

Online resources

(171) BTEC Applied Science Help - YouTube  Unit 5 All sciences

(171) BTEC Applied Science Unit 5 Revision - YouTube  Unit 5 All sciences

BTEC Applied Science Unit 5 Flashcards | Quizlet Unit 5 All sciences

Unit 5 Btec Flashcards & Quizzes | Brainscape Unit 5 All sciences

Content to be covered

Physics

C Thermal physics, materials and fluids

C1 Thermal physics in domestic and industrial applications

• Be able to use the following quantities and units:

o power, watt (W), kilowatt (kW), megawatt (MW), gigawatt (GW)

o convert °C to K

o pressure (Pascals (Pa), Newton per metre squared (Nm−2)).

• Know the following definitions:

o work done as energy transferred

o work done as force × distance moved in direction of force (W = F × ∆x)

o work done by a gas as pressure × change in volume of gas (W = p × ∆V)

• Be able to calculate efficiency using the relationships:

o efficiency = useful energy output / total energy input

o specific heat capacity from (∆Q = mc∆T)

C2 Materials in domestic and industrial applications

• Understand the following concepts and apply them in domestic and industrial applications:

o elasticity

o stress-strain curves

o elastic limit

o strength

o yield point

o plastic deformation

o creep

o fatigue

o ductility

o brittleness

o malleability

o elastic hysteresis.

# Topic 1: Thermal Physics

## GCSE Retrieval

### Q1

Define temperature.

### Q2

State the unit of temperature used in science.

### Q3

What is meant by thermal energy?

# Revision Notes

## Temperature Scales

### Celsius Scale

Water freezes at 0°C

Water boils at 100°C

### Kelvin Scale

Used in scientific calculations.

0 K = Absolute Zero

Relationship:

K = °C + 273

Examples:

20°C = 293 K

100°C = 373 K

## Practice Questions

### Q4 (2 marks)

Convert 25°C to Kelvin.

### Q5 (2 marks)

Convert 100°C to Kelvin.

### Q6 (2 marks)

Convert 300 K to °C.

## Worked Example

Convert 35°C to Kelvin.

K = °C + 273

K = 35 + 273

K = 308 K

Answer = 308 K

## Exam Question (4 marks)

Explain why scientists prefer the Kelvin scale for thermodynamic calculations.

# Topic 2: Specific Heat Capacity

## Revision Notes

Specific heat capacity is the energy required to raise the temperature of 1 kg of a substance by 1°C (or 1 K).

Equation:

Q = mcΔT

Where:

Q = Energy (J)

m = Mass (kg or g)

c = Specific heat capacity

ΔT = Temperature change

## Worked Example

Mass = 0.5 kg

c = 4200 J kg⁻¹ °C⁻¹

Temperature increase = 20°C

Q = mcΔT

Q = 0.5 × 4200 × 20

Q = 42,000 J

Answer = 42 kJ

## Practice Questions

### Q7 (3 marks)

Calculate energy required to heat 1 kg of water by 15°C.

### Q8 (3 marks)

Calculate energy required to heat 2 kg of water by 10°C.

### Q9 (4 marks)

Explain why water is used as a coolant in industry.

## Data Question (5 marks)

A student heats 500 g of water.

| Energy Added (J) | Temperature (°C) |
| --- | --- |
| 0 | 20 |
| 5000 | 22 |
| 10000 | 25 |
| 15000 | 27 |
| 20000 | 30 |

Describe the relationship shown.

# Topic 3: Work Done

## GCSE Retrieval

### Q10

State the equation for work done.

# Revision Notes

Work done occurs when a force causes movement.

Equation:

W = F × d

Where:

W = Work Done (J)

F = Force (N)

d = Distance (m)

## Worked Example

Force = 50 N

Distance = 10 m

W = Fd

W = 50 × 10

W = 500 J

Answer = 500 J

## Practice Questions

### Q11 (3 marks)

Calculate work done when:

Force = 30 N

Distance = 8 m

### Q12 (3 marks)

Calculate work done when:

Force = 120 N

Distance = 4 m

## Pressure-Volume Work

For gases:

W = pΔV

Where:

p = Pressure

ΔV = Change in volume

## Application Question (4 marks)

Explain how work is done when a gas expands.

# Topic 4: Efficiency

## Revision Notes

Efficiency measures how effectively energy is transferred.

Efficiency

= Useful Energy Output ÷ Total Energy Input

Efficiency × 100 gives percentage efficiency.

## Worked Example

Useful energy = 600 J

Input energy = 1000 J

Efficiency

= 600 ÷ 1000

= 0.60

= 60%

## Practice Questions

### Q13 (2 marks)

Calculate efficiency:

Useful = 800 J

Input = 1000 J

### Q14 (2 marks)

Calculate efficiency:

Useful = 450 J

Input = 900 J

## Extended Question (6 marks)

Evaluate why no machine is 100% efficient.

# Topic 5: Materials and Stress-Strain Curves

## Revision Notes

Stress-Strain curves show how materials behave under load.

### Stress

Force per unit area.

### Strain

Extension divided by original length.

## Key Terms

### Elasticity

Returns to original shape.

### Elastic Limit

Maximum stress before permanent deformation.

### Yield Point

Start of plastic deformation.

### Plastic Deformation

Permanent change in shape.

### Strength

Ability to withstand force.

### Ductility

Ability to stretch without breaking.

### Brittleness

Breaks with little deformation.

### Malleability

Ability to be shaped.

### Fatigue

Weakening due to repeated loading.

### Creep

Slow deformation under constant load.

## Knowledge Check

Match the term with the definition.

| Term | Definition |
| --- | --- |
| Fatigue | __________ |
| Creep | __________ |
| Elastic Limit | __________ |
| Ductility | __________ |

## Data Analysis

A steel wire is stretched.

| Force (N) | Extension (mm) |
| --- | --- |
| 10 | 1 |
| 20 | 2 |
| 30 | 3 |
| 40 | 4 |
| 50 | 7 |

### Q15 (4 marks)

Identify the anomalous result and explain your reasoning.

## Exam Question (6 marks)

Explain the difference between elastic deformation and plastic deformation.

## Extended Response (6 marks)

Evaluate why engineers must consider creep and fatigue when selecting materials.

# End of Physics Assessment

## Section A Retrieval (10 marks)

- Define specific heat capacity.

- State the equation for work done.

- State the equation for efficiency.

- Define stress.

- Define strain.

## Section B Calculations (10 marks)

### Q16

Calculate:

m = 0.8 kg

c = 4200

ΔT = 25°C

(3 marks)

### Q17

Calculate work done:

F = 75 N

d = 12 m

(3 marks)

### Q18

Calculate efficiency:

Useful = 900 J

Input = 1200 J

(4 marks)

## Section C Application (5 marks)

### Q19

Explain how specific heat capacity influences industrial heating systems.

(5 marks)

## Section D Extended Response (10 marks)

### Q20

A company is selecting a material for a bridge.

Evaluate the importance of:

- strength

- elasticity

- fatigue

- creep

in the selection process.

(10 marks)

# Physics Section Total

| Topic | Marks |
| --- | --- |
| Thermal Physics | 15 |
| Specific Heat Capacity | 15 |
| Work Done | 10 |
| Efficiency | 10 |
| Materials | 20 |
| End Assessment | 35 |

Total = 105 Marks